# Statement of Work

# Huron NF, 2022 Boundary Surveys

# GNA-Huron-FY22-ORV Aspen – Huron ORV RP

- GOVERNMENT REPRESENTATIVE AND SUBMITTAL:  Please review the Statement of Work below, along with the attached maps and respond with a not-to-exceed Quotation for this project.  Also, please note the Delivery Date in Section G below.  We request this information by close of business on March 10, 2022.  Send this to the Contracting Officers Representative (COR): Matthew Dontz by email (preferred .pdf or .docx file) at dontzm@michigan.gov .  The Contractor may request a clarification/scoping meeting (or conference call) to better define the project before preparing the submittal. The work on this project must be done according to the specifications of the contract that was previously provided; pertinent paragraphs contained therein should be carefully examined before this proposal is submitted.

- PURPOSE:  The purpose of this survey is to delineate the common boundary between National Forest and private land for upcoming management activities. This project will be named the “GNA-Huron-FY22 – ORV Aspen – Huron ORV RP”.

- PROJECT AREA:  This project consists of One (1) Project Area (shown on the attached map), located in Iosco County, Michigan, more particularly described as:

ORV Aspen – Huron ORV RP: T24NR06E Sections 5 & 26, Iosco County, Michigan

CONTROL

Section 5 so far researched does not show much recorded control other than E3 from USFS records.  Research at Court House has not been done for either section.  Section 26 has good recorded control per Remon and some previous work done in section 26 by the USFS.

ACCESS

Section 5 has a Forest Service Road running SE to NW across the section giving access and there are some smaller trails in the south half of the section and access to the NW corner from road from the north.  Section 26 has good access from county and state roads on the south and west sides of the section.  The east side has limited access and river makes access more challenging.

SURVEY RECORDS

Attached, please find the records that have been made available to the Michigan DNR for this project. It is anticipated that additional research and additional records maybe available from other sources. All survey records research is the contractor’s responsibility.

- ITEMS OF WORK:

LEGAL DESCRIPTIONS:

T24N, R06E,

Section 5: All

Road across NW NW is permitted for private access, centerline and width of road from the west to north section line shall be located and shown on drawing.

Section 26: E ½ NE, Part of NW NE (see deed L. 769, Pg 771 and survey L. 592, pg 820 for full description), SE ¼, SW ¼, NW ¼ EXCEPT a parcel (see deed L. 290, Pg 59-60 for full description)

Subject to right-of-way for M-65 and Consumers Power Electric Transmission Line Easement per L. 290, Pg 67-68.

Note that a new M-65 bridge was constructed over the Au Sable and MDOT r/w maps do not reflect updated r/w in Section 26 (see 2003 plans for alignment)

*All roads and utilities under permit or easement shall be tied and shown on survey at a minimum where they cross section and NFS property lines or as otherwise noted. All encroachments on NFS lands shall be tied and shown on survey.

- All 1/16th corners use the County Remonumentation Corner Code system

ORV Aspen – Huron ORV RP: T24NR06E Sections 5 & 26 – Items of Work will include;

Item 2 - Control Corner Measurement (10 total)

Section 5:  4

Section 26:  6

Item 3B – Monumentation of Section Subdivisional Corners (5 total): See attached map.

Sections :  1/16th corners, 1/64th corners, misc. corners (4 total) – See map.

Item 6 and 7 – Property Line Measurement and Property Line Posting (2.50 miles total):

Section 5:   1.50 miles

Section 26:  1.00 miles

Item 9A and 9B – Certificate of Survey (1 total, section 26)

E.	Notification letters to adjoining private landowners must be sent prior to commencing field work in the interior of sections.

F.  ENCROACHMENTS:  The extent of any encroachments discovered shall be brought to the attention of the Contracting Officers Representative upon confirmation by the property line survey.

G.	DELIVERY DATE:	The final project submission delivery date: 07/31/2022

Attachments:   	-Project maps

-USFS Specifications

-Exhibits

-State of Michigan Terms and Conditions

Matthew S. Dontz PS 4001056128

Michigan Department of Natural Resources & Contracting Officers Representative (COR)

07-28-2021