# Statement of Work

# Michigan DNR, 2022 Boundary Surveys

# FRD-Gaylord Unit-FY23-Project#4

- GOVERNMENT REPRESENTATIVE AND SUBMITTAL:  Please review the Statement of Work below, along with the attached maps and respond with a not-to-exceed Quotation for this project.  Also, please note the Delivery Date in Section G below April 13th, 2023. We request this information by close of business on [SEE POSTING]. The Contractor may request a clarification/scoping meeting (or conference call) to better define the project before preparing the submittal. The work on this project must be done according to the specifications of the contract that was previously provided; pertinent paragraphs contained therein should be carefully examined before this proposal is submitted.

- PURPOSE:  The purpose of this survey is to delineate the common boundary between the State of Michigan land and private land for upcoming management activities. This project will be named the “FRD-Gaylord Unit-FY23-Project#4”.

- PROJECT AREA:  This project consists of One (1) Sub Project Areas (shown on the attached maps), located in Charlevoix County, Michigan, more particularly described as:

Sub Project Area #4: T33N-R05W Sections 22,28 & 33, Charlevoix County

CONTROL

Good part of township was surveyed in 1982 by Department Surveyor Carl Roser.  Remon has records of north and west line of section 21 and W ¼ corner of section 28.  Control by Roser appears to be accepted.

ACCESS

Access could be challenging on some parts of this project.  Even though US-131 runs

N-S though approximate center of project giving some access, due to topography and lack of county roads near or on section or quarter lines.  Some two-track trail roads are available.

SURVEY RECORDS

Attached, please find the records that have been made available by the Michigan DNR for this project. An index sheet shows what records of corner records are held in house by the Department that can be obtained if requested along with legal descriptions of State Owned Lands.  It is anticipated that additional research and additional records maybe available from other sources. All survey records research is the contractor’s responsibility.

- ITEMS OF WORK:

- All 1/16th corners use the County Remonumentation Corner Code system

Sub Project Area #4: T33N-R05W Sections 22, 28 & 33 – Items of Work will include;

Item 1 - Corner Search (1 total): Possibly C ¼ of Section 22

[Corners don’t have a current LCRC or Monument of Record; may require a monument to be placed along with witnessing, drafting a LCRC, and recordation of said certificate]

Item 2 – Control Corner Measurement (16 total):

[Corners with a LCRC; may or may not require a monument to be placed, may require rewitnessing, may require drafting a LCRC, and may require recordation of said certificate]

Section 22:  (3 corners)

Section 28:  (4 corners)

Section 33: (9 corners)

Item 3 – Corner Monumentation

Item 3B – Monumentation of Section Subdivisional Corners (10 total):

[Corners don’t have a current LCRC; may require a monument to be placed, along with witnessing, drafting a LCRC, and recordation of said certificate]

Section 22:  Find or Establish C ¼ corner.  (1 corner)

Section 28:  2,6,7 (3 corners)

Section 33: 3,5,8,14,15, 5 misc. corners (10 corners)

Item 9A and 9B – Certificate of Survey/ Plat of Survey - Drawing (3 total)

[Map depicting the findings of this survey; vendor may provide a recordable map consisting of multiple sheets OR a non-recordable Large format drawing]

E.	NOT REQUIRED FOR THIS PROJECT Notification letters to adjoining private landowners must be sent prior to commencing field work in the interior of sections.

F.  ENCROACHMENTS:  The extent of any encroachments discovered shall be brought to the attention of the Contracting Officers Representative upon confirmation by the property line survey.

G.	DELIVERY DATE:	The final project submission delivery date: April 13th, 2023

Attachments:   	-Project maps

-MDNR Specifications & Exhibits

-State of Michigan Terms and Conditions

Matthew Dontz,  PS 56128

Michigan Department of Natural Resources & Contracting Officers Representative (COR)

10-22-2022